TAC 2026

Dual-Camera Vision System

Technical Documentation & Command Reference

Files Covered:

tac2026-fixed.py   |   underwater_enhance.py

# 1. Project Overview

The TAC 2026 system is an all-in-one dual-camera underwater vision pipeline designed for autonomous underwater vehicle (AUV) competitions. It provides real-time ArUco marker detection, adaptive image enhancement, pipeline tracking, and mission state management across two camera feeds.

## 1.1 System Architecture

The system supports three operational modes:

- Laptop-only mode: Both cameras connect directly to the laptop. No GStreamer or UDP streaming involved.

- Transmit mode (run on Jetson Nano/AGX): Captures from both cameras and streams via H.264/RTP over UDP to a laptop.

- Receive mode (run on Laptop): Receives UDP streams from the Jetson, decodes via GStreamer, and runs all vision processing.

## 1.2 File Summary

| File | Role | Key Contents |
| --- | --- | --- |
| tac2026-fixed.py | Main application | Arg parsing, lighting, ArUco detection, pipeline mask, HUD, transmitter, receiver loop |
| underwater_enhance.py | Enhancement stub | Camera matrices, distortion coefficients, enhance_for_aruco() function |

# 2. tac2026-fixed.py — Detailed Documentation

## 2.1 Module-Level Imports & Dependencies

| Library | Purpose |
| --- | --- |
| cv2 (OpenCV) | Camera capture, image processing, ArUco detection, GUI windows |
| cv2.aruco | ArUco marker detection, dictionary management, detector parameters |
| numpy (np) | Matrix math, LUTs, image array manipulation |
| threading | Concurrent ArUco worker threads and multi-camera receivers |
| subprocess | Launch GStreamer pipeline as external process (transmitter) |
| queue | Thread-safe frame and result queues between workers |
| argparse | CLI argument parsing |
| rclpy (optional) | ROS 2 integration (gracefully skipped if not installed) |
| underwater_enhance | Camera matrices, distortion coeffs, enhance_for_aruco() |

## 2.2 Argument Parsing — parse_args()

Parses command-line arguments. The three mode flags (--laptop, --receive, --transmit) are mutually exclusive.

| Argument | Type | Default | Description |
| --- | --- | --- | --- |
| --laptop | flag | — | Run TX + RX together on laptop (direct camera access) |
| --receive | flag | — | Receive-only mode; Jetson streams via Ethernet |
| --transmit | flag | — | Transmit-only; run on Jetson, streams to laptop |
| --host <ip> | string | None | Laptop IP address (required when --transmit is used) |
| --dev0 <device> | string | /dev/video0 | Camera 0 device path |
| --dev1 <device> | string | /dev/video2 | Camera 1 device path |
| --bitrate <kbps> | int | 5000 | H.264 encoding bitrate in kbps |
| lighting (positional) | choice | AUTO | Initial lighting preset: bright, midday, evening, night, auto |

## 2.3 Lighting Profiles — LIGHTING_MODES

Four named lighting profiles are defined, each tuning a set of image processing parameters. These are applied by the adaptive enhancement pipeline.

| Profile | gamma | red_gain | CLAHE clip | CLAHE tile | Best for |
| --- | --- | --- | --- | --- | --- |
| BRIGHT | 0.7 | 1.1 | 2.0 | 8 | Bright outdoor / surface-level water |
| MIDDAY | 1.0 | 1.3 | 3.0 | 8 | Normal daylight, moderate depth |
| EVENING | 1.3 | 1.5 | 3.5 | 6 | Dim light, late afternoon / low sun |
| NIGHT | 1.8 | 1.7 | 5.0 | 4 | Very dark conditions, deep water |

Each profile also contains: wb_strength (white-balance aggressiveness), deblur_s (unsharp-mask strength), sharpen_thr_lo/hi (Laplacian sharpness thresholds), auto_lo/hi (brightness range for AUTO detection), color_balance (per-channel BGR offset), and button colors for the HUD.

## 2.4 LightingState Class

A thread-safe class managing the current lighting mode and per-mode gamma/exposure overrides.

| Method | Description |
| --- | --- |
| __init__(initial) | Initialises with a mode string ('AUTO' or a named profile) |
| set_mode(m) | Set active lighting mode (thread-safe) |
| get_mode() | Return current mode string |
| toggle_tac() | Toggle TAC (enhanced dehazing) on/off; returns new state |
| tac_on() | Returns True if TAC mode is active |
| adjust_gamma(d, r) | Add delta d to gamma override for profile r |
| adjust_exposure(d, r) | Add integer delta to exposure_shift override for profile r |
| reset_mode(r) | Reset all overrides for profile r back to defaults |
| resolve(brt) | Return the best profile name for brightness value brt |
| profile(r) | Return the resolved profile dict for r with overrides applied |

## 2.5 Camera Calibration

Two cameras are hardcoded with intrinsic matrices and distortion coefficients. Undistortion maps are pre-computed at startup using cv2.initUndistortRectifyMap() for fast per-frame remapping.

| Camera | Focal Length (px) | Principal Point | Distortion Model |
| --- | --- | --- | --- |
| CAM0 (HP Wide) | fx=fy=600 | cx=320, cy=240 | 5-coeff: k1=-0.30, k2=0.10 |
| CAM1 (Lenovo) | fx=fy=554.256 | cx=320, cy=240 | 5-coeff: k1=-0.38, k2=0.15, k5=-0.035 |

## 2.6 ArUco Configuration

Four ArUco dictionaries are searched in parallel: 4x4, 5x5, 6x6, 7x7 (all with 1000 IDs). Detection runs on a 50% downscaled frame (DETECT_SCALE=0.5) for speed.

### Pipeline Tags (special IDs)

| Tag ID | Order | Name | Message |
| --- | --- | --- | --- |
| 56 | 1 | PIPELINE ENTRY | Pipeline detected — move to next section |
| 5 | 2 | SECTION 2 | Section 2 reached — proceed forward |
| 20 | 3 | SECTION 3 | Section 3 reached — scan surroundings |
| 32 | 4 | PIPELINE EXIT | Pipeline exit — inspection complete |

### Key Detection Parameters

| Parameter | Value | Description |
| --- | --- | --- |
| CENTER_TOLERANCE | 60 px | Pixels from center considered 'locked' |
| MAX_LOST | 8 frames | Frames before target considered lost |
| MIN_MARKER_AREA | 500 px² | Minimum marker area after scaling |
| MARKER_RESULT_TTL | 0.10 s | Time-to-live for a detection result |
| SPATIAL_MERGE_DIST | 80 px | Distance to merge duplicate detections |
| DETECT_SCALE | 0.5 | Downscale factor for detection frame |

## 2.7 Image Enhancement Pipeline — enhance_adaptive()

Applied every frame to the raw camera image before display and pipeline detection.

| Step | Operation | Notes |
| --- | --- | --- |
| 1. Undistort | cv2.remap() with precomputed maps | Corrects lens distortion |
| 2. Median blur | cv2.medianBlur(out, 3) | Removes salt-and-pepper noise |
| 3. Exposure shift | np.clip(out + exposure_shift) | Applied if non-zero override |
| 4. White balance | Per-channel gain based on mean brightness | Weighted by wb_strength and red_gain |
| 5. Color balance | Per-channel BGR additive offset | From profile color_balance tuple |
| 6. Gamma LUT | cv2.LUT() with prebuilt table | Non-linear brightness correction |
| 7. CLAHE | Applied to L channel in LAB space | Adaptive histogram equalisation |
| 8. TAC mode (optional) | _dehaze() + extra CLAHE (clip=5, tile=4) | Activated by T key or TAC button |
| 9. Unsharp mask | cv2.filter2D() with deblur kernel | Strength set by profile deblur_s |
| 10. Adaptive sharpen | Laplacian score → addWeighted() | 2.0x if score < lo, 1.3x if < hi, else skip |

## 2.8 Pipeline Mask — compute_pipeline_mask()

Detects the orange/yellow pipeline object in the frame using HSV colour segmentation.

| Parameter | Value | Description |
| --- | --- | --- |
| HSV Range 1 | H:18-40, S:90-255, V:70-255 | Orange-yellow band |
| HSV Range 2 | H:5-18, S:100-255, V:60-255 | Deep orange band |
| PIPE_MIN_AREA | 800 px² | Minimum contour area |
| PIPE_MIN_ASPECT | 2.2 | Minimum length-to-width ratio (elongated shape) |
| PIPE_MIN_FILL_RATIO | 0.45 | Minimum area / bounding rect fill |
| PIPE_MIN_SOLIDITY | 0.65 | Minimum area / convex hull ratio |
| PIPE_MIN_LENGTH | 80 px | Minimum long-axis length |

Returns: mask visualisation frame, pipeline_found bool, angle (degrees), centroid (cpx, cpy).

## 2.9 ArUco Worker Threads — make_aruco_worker()

Four daemon threads run in parallel, one per ArUco dictionary. Each reads downscaled frames from its own input queue, detects markers, and pushes results to a shared result queue. Priority ordering (1–4 mapped to 4x4–7x7) resolves conflicts when the same physical marker is detected by multiple dictionaries.

## 2.10 UI & HUD Components

| Component | Class / Function | Description |
| --- | --- | --- |
| Toast overlay | ToastMessage | Timed centred overlay for mode changes and tag events (3 s duration, fade out) |
| Mission state | MissionState | Tracks seen tags, sequence, pipeline found/angle, pinger side |
| Panel backdrop | draw_panel() | Semi-transparent dark rectangle for HUD areas |
| Data row | draw_row() | Two-column label:value text row |
| Crosshair | draw_crosshair() | Circle + lines at frame centre |
| Button bar | draw_button_bar() | Two rows of clickable mode and control buttons at bottom |
| EXP trackbar | cv2.createTrackbar('EXP') | Range 0–200, centre=100 (maps to -100…+100 exposure shift) |
| GAM trackbar | cv2.createTrackbar('GAM') | Range 0–270, default=100 (maps to gamma 0.30…3.00) |

## 2.11 Transmitter — run_transmitter()

Launches a GStreamer subprocess to stream one camera over UDP/RTP as H.264.

gst-launch-1.0 v4l2src device=<device> do-timestamp=true

! video/x-raw,width=640,height=480,framerate=30/1

! queue max-size-buffers=1 leaky=downstream

! videoconvert

! videobalance brightness=<b> contrast=<c>

! x264enc tune=zerolatency speed-preset=veryfast bitrate=<kbps>

key-int-max=60 bframes=0

! rtph264pay config-interval=1 pt=96

! udpsink host=<laptop_ip> port=<5000|5001> sync=false async=false

| Camera | Port | Brightness | Contrast |
| --- | --- | --- | --- |
| CAM0 | 5000 | 0.050 | 1.100 |
| CAM1 | 5001 | 0.080 | 1.150 |

## 2.12 Receiver Loop — run_receiver()

The main per-camera processing loop. Handles both direct capture (laptop mode) and GStreamer UDP receive (Jetson mode).

# 3. underwater_enhance.py — Detailed Documentation

This module serves as a calibration and enhancement stub that is imported by tac2026-fixed.py. It exposes camera matrices, distortion coefficients, and a pre-processing function.

## 3.1 Exported Symbols

| Symbol | Type | Description |
| --- | --- | --- |
| CAMERA_MATRICES | dict[int → np.ndarray (3x3)] | Intrinsic matrix for cam 0 and cam 1 |
| DIST_COEFFS | dict[int → np.ndarray (5,)] | Distortion coefficients for cam 0 and cam 1 |
| enhance_for_aruco() | function | Pre-processes a frame for better ArUco detection |

## 3.2 Camera Matrices

CAMERA_MATRICES = {

0: [[600.,    0.,  320.],    # CAM0 — HP Wide

[  0.,  600.,  240.],

[  0.,    0.,    1.]],

1: [[554.256,    0.,  320.], # CAM1 — Lenovo

[  0.,  554.256,  240.],

[  0.,       0.,    1.]]

}

Replace these placeholder values with real calibration results from cv2.calibrateCamera() or similar tools.

## 3.3 Distortion Coefficients

DIST_COEFFS = {

0: [-0.30,  0.10, 0., 0.,  0.    ],  # k1, k2, p1, p2, k3

1: [-0.38,  0.15, 0., 0., -0.035 ],  # k1, k2, p1, p2, k3

}

## 3.4 enhance_for_aruco(frame, cam_id, tac_mode=False)

Pre-processes a raw BGR frame to improve ArUco detection reliability underwater.

| Step | Operation | Normal mode | TAC mode extra |
| --- | --- | --- | --- |
| 1 | Convert to grayscale | cv2.COLOR_BGR2GRAY | — |
| 2 | Apply CLAHE | clipLimit=3.0, tile=(8,8) | — |
| 3 | Sharpening | skipped | 3x3 sharpening kernel applied |
| 4 | Output | Convert gray back to BGR | Convert gray back to BGR |

This function returns a BGR image (not grayscale) so it integrates cleanly into the existing colour pipeline.

This is a stub — replace enhance_for_aruco() with domain-specific underwater enhancement logic for best results in your specific water conditions.

# 4. Commands Reference

## 4.1 Run Commands

### Condition 1 — Laptop Only

Use when both cameras are connected directly to the laptop (no Jetson).

python3 tac2026.py --laptop

python3 tac2026.py --laptop bright

python3 tac2026.py --laptop night

python3 tac2026.py --laptop --dev0 /dev/video0 --dev1 /dev/video2

### Condition 2 — Jetson + Laptop (Ethernet Streaming)

Run the transmitter on the Jetson and the receiver on the laptop.

# On the LAPTOP (receiver):

python3 tac2026.py --receive

python3 tac2026.py --receive midday

# On the JETSON (transmitter):

python3 tac2026.py --transmit --host 192.168.1.100

python3 tac2026.py --transmit --host 192.168.1.100 --bitrate 3000

python3 tac2026.py --transmit --host 192.168.1.100 --dev0 /dev/video0 --dev1 /dev/video4

## 4.2 All CLI Options

| Command / Option | Example | Notes |
| --- | --- | --- |
| --laptop | python3 tac2026.py --laptop | Direct camera mode (no GStreamer) |
| --receive | python3 tac2026.py --receive | UDP receive mode (laptop side) |
| --transmit --host | python3 tac2026.py --transmit --host 10.0.0.5 | Stream mode (Jetson side); --host required |
| --dev0 <path> | --dev0 /dev/video4 | Override Camera 0 device (default: /dev/video0) |
| --dev1 <path> | --dev1 /dev/video6 | Override Camera 1 device (default: /dev/video2) |
| --bitrate <kbps> | --bitrate 3000 | H.264 bitrate in kbps (default: 5000) |
| lighting (positional) | python3 tac2026.py --laptop bright | One of: bright midday evening night auto |

## 4.3 Keyboard Shortcuts (Camera Window)

| Key | Action |
| --- | --- |
| Q | Quit — close all windows and stop |
| 1 | Switch to BRIGHT lighting mode |
| 2 | Switch to MIDDAY lighting mode |
| 3 | Switch to EVENING lighting mode |
| 4 | Switch to NIGHT lighting mode |
| A | Switch to AUTO lighting mode (auto-selects based on brightness) |
| T | Toggle TAC mode (extra dehazing + CLAHE) |
| [ (left bracket) | Decrease exposure (legacy — now use EXP trackbar) |
| ] (right bracket) | Increase exposure (legacy — now use EXP trackbar) |
| - (minus) | Decrease gamma (legacy — now use GAM trackbar) |
| = or + (equals) | Increase gamma (legacy — now use GAM trackbar) |
| R | Reset all overrides for current mode to defaults |

## 4.4 On-Screen Controls

| Control | Type | Description |
| --- | --- | --- |
| BRIGHT / MIDDAY / EVENING / NIGHT / AUTO | Buttons (Row 1) | Click to switch lighting mode |
| TAC: ON/OFF | Button (Row 2) | Toggle TAC enhanced dehazing |
| RESET | Button (Row 2) | Reset current mode overrides to defaults |
| QUIT | Button (Row 2) | Exit the application |
| EXP trackbar | Slider (0–200, centre=100) | Adjusts exposure offset for current mode (delta from slider) |
| GAM trackbar | Slider (0–270, default=100) | Adjusts gamma for current mode (value * 0.01 + 0.30) |

## 4.5 GStreamer Pipeline (Transmitter)

# CAM0 → laptop port 5000

gst-launch-1.0 v4l2src device=/dev/video0 do-timestamp=true \

! video/x-raw,width=640,height=480,framerate=30/1 \

! queue max-size-buffers=1 leaky=downstream \

! videoconvert \

! videobalance brightness=0.050 contrast=1.100 \

! x264enc tune=zerolatency speed-preset=veryfast bitrate=5000 \

key-int-max=60 bframes=0 \

! rtph264pay config-interval=1 pt=96 \

! udpsink host=<LAPTOP_IP> port=5000 sync=false async=false

# CAM1 → laptop port 5001

gst-launch-1.0 v4l2src device=/dev/video2 do-timestamp=true \

! video/x-raw,width=640,height=480,framerate=30/1 \

! queue max-size-buffers=1 leaky=downstream \

! videoconvert \

! videobalance brightness=0.080 contrast=1.150 \

! x264enc tune=zerolatency speed-preset=veryfast bitrate=5000 \

key-int-max=60 bframes=0 \

! rtph264pay config-interval=1 pt=96 \

! udpsink host=<LAPTOP_IP> port=5001 sync=false async=false

## 4.6 GStreamer Pipeline (Receiver / Laptop Side)

# CAM0 receive on port 5000

udpsrc port=5000 \

caps="application/x-rtp, media=video, encoding-name=H264, payload=96" \

! rtpjitterbuffer latency=200 \

! rtph264depay ! h264parse ! avdec_h264 \

! videoconvert \

! video/x-raw, format=BGR \

! appsink drop=True max-buffers=1 sync=false

Replace port=5000 with port=5001 for the second camera. This pipeline string is constructed automatically by run_receiver() — manual use is for diagnostics only.

# 5. Dependencies & Setup

## 5.1 Python Packages

| Package | Version | Install |
| --- | --- | --- |
| opencv-python (cv2) | 4.x recommended | pip install opencv-python |
| opencv-contrib-python | 4.x (for aruco) | pip install opencv-contrib-python |
| numpy | any recent | pip install numpy |
| rclpy (optional) | ROS 2 Humble+ | ROS 2 install (skip for non-ROS use) |

## 5.2 System Requirements

| Component | Requirement |
| --- | --- |
| GStreamer | gstreamer1.0-tools, gst-plugins-good, gst-plugins-bad, gst-plugins-ugly |
| x264 | libx264-dev (for H.264 encoding on Jetson) |
| avdec_h264 | gstreamer1.0-libav (for decoding on laptop) |
| Camera | V4L2-compatible USB cameras on Linux (e.g. /dev/video0) |
| Python | 3.8 or newer |

## 5.3 Calibration — Replacing Stub Values

The values in underwater_enhance.py are placeholder estimates. To replace them:

- Capture a checkerboard pattern from each camera (minimum 20 images per camera).

- Run cv2.calibrateCamera() to obtain the intrinsic matrix and distortion coefficients.

- Replace CAMERA_MATRICES[0], CAMERA_MATRICES[1], DIST_COEFFS[0], DIST_COEFFS[1] in underwater_enhance.py.

- Also update CAM0_MATRIX, CAM0_DIST, CAM1_MATRIX, CAM1_DIST in tac2026-fixed.py (used for the undistort maps).

Both files currently share the same placeholder values. Keep them in sync after calibration, or refactor tac2026-fixed.py to import calibration from underwater_enhance.py.

End of Documentation